Week 6 Lab Task

Name: Faseeha Siddiqui

Network Sniffing & WiFi Hacking

NIAI NAVTTC — Certified Ethical Hacker (CEH) Course

Task Part 1: A small business is experiencing network connectivity problems causing disruptions to their operations. Use sniffing techniques to diagnose the root cause and propose solutions.

Task Part 2: Conduct a wireless security assessment for a client's premises. Identify any unauthorized or hidden wireless networks that could pose security risks.

PART 1 — Network Sniffing (Diagnose Connectivity Issues)

▶  Set Up Your Lab Network

Make sure Kali Linux and Metasploitable 2 are running on the same Host-Only network.

Both machines should be able to ping each other before you start.

Note your Kali IP (ifconfig) and Metasploitable IP — you will need these.

Metasploitable IP: 192.168.224.101

Kali Ip: 192.168.224.102

▶  Capture Traffic with Wireshark (GUI)

Open Wireshark from Kali Applications → Sniffing & Spoofing → Wireshark.

Select your network interface (usually eth0 or the Host-Only adapter).

Click the blue shark-fin Start button to begin capturing.

Generate some traffic: open a browser, ping Metasploitable, browse its web server.

Stop the capture after 1-2 minutes.

▶  Analyse the Capture in Wireshark

Use the filter bar at the top to isolate specific traffic:

http                    # Show only HTTP traffic

tcp                     # Show only TCP packets

icmp                    # Show only ping packets

ip.addr == 192.168.x.x  # Filter by specific IP address

Look for: retransmissions (sign of packet loss), connection resets (RST flags), slow responses, or unusual protocols.

Right-click any TCP stream → Follow → TCP Stream to read full conversation.

HTTP:

TCP:

ICMP:

IP address search

TCP Stream

▶  Capture Traffic with tcpdump (Command Line)

Open a terminal in Kali and run:

sudo tcpdump -i eth0                          # Capture all traffic

sudo tcpdump -i eth0 -w capture.pcap          # Save to file

sudo tcpdump -i eth0 host 192.168.56.101      # Filter by host

sudo tcpdump -i eth0 port 80                  # Filter by port

wireshark capture.pcap                        # Open saved capture in Wireshark

Now capture traffic in Wireshark while the MitM is active. You will see both sides of the conversation.

When done, stop arpspoof and reset ip_forward to 0.

▶  Diagnose & Document Findings

Based on your packet capture, write a diagnosis report:

- What protocols were seen on the network?

The following protocols were observed in the Wireshark capture:

- HTTP → Web communication with the Metasploitable server

- TCP → Transport layer protocol managing connections

- ICMP → Ping requests and replies between hosts

- ARP → Resolving IP addresses to MAC addresses

- NBNS / BROWSER → Network discovery and workgroup announcements

- Were any credentials or sensitive data visible in plaintext?

- Yes, HTTP traffic was visible in plaintext.

- The full HTTP request and response could be read using “Follow TCP Stream”.

- Exposed information included:

- Webpage content (HTML)

- Server details (Apache version, PHP version)

- No usernames or passwords were captured in this specific traffic, but:

- Since HTTP is unencrypted, any login credentials would be exposed if transmitted.

- What is the likely root cause of the connectivity problem?

- No major network issues were detected:

  - ICMP replies were consistent → no packet loss

  - TCP connections completed successfully

  - No retransmissions or reset (RST) packets observed

Therefore, the likely conclusion is:

The network itself is stable and the problem is likely due to:

  - Server-side performance issues

  - Application-level delays

  - Misconfiguration rather than actual connectivity failure

- What solutions do you recommend? (e.g. use HTTPS, switch to SSH, fix DNS config)

1) Security Improvements

- Use HTTPS instead of HTTP to encrypt data

- Disable unnecessary services and ports

- Hide server version information

2)  System Improvements

- Update outdated software:

  - Apache server

  - PHP version

- Apply security patches

3) Network Improvements

- Monitor traffic regularly using tools like Wireshark

- Reduce unnecessary broadcast traffic

- Ensure proper network configuration

4)  Protocol Improvements

- Replace insecure protocols:

  - HTTP →  HTTPS

  - Telnet → SSH